HoloAssist

Technical Documentation

XR Teleoperation & Collaboration Framework  —  Meta Quest 3 + UR3e + OnRobot RG2

Sprint 4    |    Version 1.0    |    2026-05-13    |    github.com/John-A-Chen/HoloAssist

| Team Member | Subsystem |
| --- | --- |
| John Chen | Perception |
| Oliver Lau | Autonomous Control |
| Nicholas Sabatta | Teleoperation and Interaction |
| Sebastian Baudille | XR Scene and Visualisation |

Table of Contents

Right-click in Word and select 'Update Field' to populate page numbers.

1.  Project Overview

HoloAssist is a real-time Extended Reality (XR) teleoperation and collaboration framework integrating Meta Quest 2/3 headsets with a collaborative UR3e robot arm and OnRobot RG2 gripper. The system enables intuitive human-robot collaboration by combining XR visual overlays (live robot data, bin sorting status, TF axis visualisation) with real-time robot control via Jacobian-based velocity commands, MoveIt 2 trajectory planning, and mixed-reality operator feedback.

1.1  Operating Modes

| Mode | Description | Primary Interface |
| --- | --- | --- |
| Teleoperation | Operator wears Quest 3 and directly controls the UR3e via RMRC, Direct Joint, or Hand Guide | Meta Quest 3 controllers + RobotController.cs |
| Autonomous | Robot autonomously sorts detected cubes into bins using MoveIt 2; no operator input | ros2 service call /holoassist/pick_cube_to_bin ... |

1.2  Key Features

| Feature | Description |
| --- | --- |
| AprilTag Perception | RealSense D435i + apriltag_ros detects workspace board and 4 cubes; metric poses in workspace_frame |
| MoveIt 2 Autonomous Sorting | Collision-free pick-and-place: Cartesian approach + OMPL fallback; configurable bin destinations |
| XR Teleoperation (3 modes) | RMRC (DLS Jacobian), Direct Joint, Hand Guide; 50 Hz joint velocity via ros_tcp_endpoint |
| XR Heads-Up Display | RobotDataPanel (joints, EE pose, mode, gripper %); BinStatusPanel (per-bin count); RadialMenu (13 toggles) |
| Mixed Reality / VR toggle | PassthroughToggle switches MR (passthrough) and full VR |
| React Web Dashboard | Remote monitoring without a headset; same rosbridge data source |
| Session Logging + E-stop | SessionLogger records mode durations; PyQt5 dashboard E-stop: burst zeros + controller switch |

1.3  Subsystem List

| # | Subsystem | Owner | Package(s) |
| --- | --- | --- | --- |
| 1 | Perception | John Chen | ros2_ws/src/holo_assist_depth_tracker/ |
| 2 | Autonomous Sorting | Oliver Lau | ros2_ws/src/moveit_robot_control/ + moveit_robto_control_msg |
| 3 | Teleoperation & Interaction | Nicholas Sabatta (Nic) | Unity/Assets/Scripts/ + dashboard/ + ros2_ws/src/ROS-TCP-Endpoint/ |
| 4 | XR Scene & Visualisation | Sebastian Baudille | Unity/Assets/ (Scenes/UI) |

1.4  Coordinate Frame Reference

| Frame | Origin / Purpose | Publisher | Consumers |
| --- | --- | --- | --- |
| base_link | UR3e robot base (MoveIt planning frame) | ur_onrobot_control driver | Everything — canonical reference |
| workspace_frame | Workspace board origin (AprilTag corner 0) | holoassist_workspace_board_node | Perception, Sorting adapter, RViz |
| camera_color_optical_frame | RealSense D435i optical centre | RealSense driver | Perception, calibration |
| apriltag_cube_1 through _4 | Detected cube centres from face-tag consensus | holoassist_cube_pose_node | Sorting adapter, RViz |
| tool0 | UR3e end-effector frame | robot_state_publisher | Teleop Jacobian, RViz |
| unity_cube | Interactive Unity cube pose (placeholder) | ROSObjectPublisher (XR subsystem) | RViz |

1.5  System Architecture Diagram

2.  Dependencies

2.1  Hardware — Bill of Materials

| Component | Model / Spec | Qty | Notes |
| --- | --- | --- | --- |
| Laptop (ROS + Unity host) | Ubuntu 22.04 LTS native; 16+ GB RAM; modern GPU | 1 | Native Ubuntu required (not WSL) for UR driver |
| XR Headset | Meta Quest 3 (primary); Quest 2 compatible | 1 | Developer Mode + USB debugging enabled |
| Robot arm | Universal Robots UR3e | 1 | IP: 192.168.0.192; External Control URCap required on teach pendant |
| Gripper | OnRobot RG2 | 1 | Modbus interface via robot controller; 0-85 mm jaw range |
| RGB-D Camera | Intel RealSense D435i | 1 | USB 3.0 (blue port); ceiling-mounted above workspace |
| Robot router | 2.4 / 5 GHz dual-band, DHCP | 1 | Provides 192.168.0.x subnet for all devices |
| Workspace board | 700 mm x 500 mm with AprilTags at corners (IDs 0-3) | 1 | Tag edge exactly 32 mm — must match config/apriltag_all.yaml |
| Cubes | Physical cubes with 6-face AprilTags (IDs 10-33 across 4 cubes) | 4 | Tag edge exactly 32 mm; cube edge 40 mm |

2.2  Computing Requirements

| Requirement | Spec |
| --- | --- |
| OS | Ubuntu 22.04 LTS native (not WSL) |
| RAM | 16+ GB |
| GPU | Modern GPU recommended for Unity XR rendering |
| Network | Wi-Fi on 192.168.0.x subnet (robot router); Ethernet at 192.168.0.1xx to UR3e |
| Headset | Meta Quest 3, Developer Mode, same Wi-Fi subnet as laptop |

2.3  Software Dependencies

| Component | Version | How to Install |
| --- | --- | --- |
| ROS 2 | Humble Hawksbill | curl + apt — see Section 3.2 |
| Unity Editor | 6.3 LTS (6000.3.9f1) | Unity Hub - Installs; add Android Build Support module |
| Meta XR All-in-One SDK | v85.0.0 | Unity Package Manager (inside project) |
| ur_onrobot_control | latest main-ros2 | git clone github.com/tonydle/OnRobot_ROS2_Driver into ros2_ws/src/ |
| ROS-TCP-Connector | v0.7.0 (local) | Clone into ROS-TCP-Connector/ at repo root; manifest.json file:// reference |
| URDF-Importer | local | Clone into URDF-Importer/ at repo root; manifest.json file:// reference |
| apriltag_ros | humble | sudo apt install ros-humble-apriltag-ros |
| RealSense SDK | humble | sudo apt install ros-humble-realsense2-camera |
| MoveIt 2 | humble | sudo apt install ros-humble-moveit |
| Easy-Handeye2 | local clone | Clone into ros2_ws/src/easy_handeye2/; colcon build --packages-select easy_handeye2 |
| Node.js | 18+ | node --version to verify; install from nodejs.org |
| React + Vite | via npm | cd holoassist-dashboard && npm install |

3.  Installation

3.1  Hardware Setup

- Mount the RealSense D435i above the workspace on a rigid stand. Connect to laptop via USB 3.0 (blue port).

- Place the workspace board (700 mm x 500 mm) in the robot workspace. Ensure all four corner AprilTags (IDs 0-3) are unobstructed.

- Place the four cubes with AprilTag faces visible to the camera (at least one face per cube).

- Connect the UR3e to the laptop via Ethernet: laptop Ethernet -> robot router; set laptop Ethernet IP to 192.168.0.100. Robot IP: 192.168.0.192.

- Connect laptop Wi-Fi to the robot router (192.168.0.x subnet). Confirm Quest 3 is on the same router.

- Power on the UR3e. On the teach pendant: load and run the External Control program (host IP: 192.168.0.100).

3.2  Software Setup

1.  Install ROS 2 Humble

curl -fsSL https://repo.ros2.org/ros.key | sudo apt-key add -

echo "deb [arch=amd64] http://repo.ros2.org/ubuntu $(lsb_release -cs) main" | sudo tee /etc/apt/sources.list.d/ros2.list

sudo apt update && sudo apt install -y ros-humble-desktop

source /opt/ros/humble/setup.bash && rosdep init && rosdep update

2.  Clone the repository and external packages

git clone https://github.com/John-A-Chen/HoloAssist.git HoloAssist_Main && cd HoloAssist_Main

git clone git@github.com:Tony-Leu/UR_OnRobot_ROS2.git ros2_ws/src/ur_onrobot_control

git clone https://github.com/Unity-Technologies/ROS-TCP-Connector.git

git clone https://github.com/Unity-Technologies/URDF-Importer.git

3.  Install ROS packages and dependencies

sudo apt install -y ros-humble-apriltag-ros ros-humble-realsense2-camera ros-humble-moveit

cd ros2_ws && rosdep install --from-paths src --ignore-src -r -y

4.  Build the ROS workspace

colcon build --symlink-install && source install/setup.bash

5.  Install the web dashboard

cd ../holoassist-dashboard && npm install

6.  Verify installation

ros2 pkg list | grep holo_assist

# Expected: holo_assist_depth_tracker    holo_assist_depth_tracker_sim

4.  Running the System

4.1  Quick Start — Virtual Test (No Robot)

# Terminal 1 — full ROS stack (fake hardware)

cd HoloAssist_Main && ./launch.sh

# Terminal 2 — E-stop dashboard (optional)

./dashboard.sh --fullscreen

# Terminal 3 — web dashboard (optional)

cd holoassist-dashboard && npm run dev   # open http://localhost:5173

# Unity: File -> Play  |  ROS Settings: IP = 127.0.0.1, Port = 10000

4.2  Real Hardware Launch

cd HoloAssist_Main && ./launch.sh --robot-ip 192.168.0.192

# On teach pendant: Load and run External Control (host IP 192.168.0.100)

# Unity ROS Settings: IP = <laptop WiFi IP e.g. 192.168.0.102>, Port = 10000

4.3  Expected Behaviour After Launch

- ros_tcp_endpoint starts on port 10000; beacon.py UDP multicast allows Quest auto-discovery.

- Unity connects; two floating panels appear in the Quest headset (joint data left, bin status right).

- Panel status shows 'Connected'; joint angles update live.

- Perception: workspace_mode reports 'locked' once all four board corner tags are visible.

- Cube poses appear on /holoassist/perception/april_cube_N_pose at ~10 Hz when cubes in view.

- Pressing Y on the left controller opens the 13-button radial menu.

[Insert screenshot of expected headset view and terminal output here]

4.4  Deploy to Quest 3

cd Unity/My\ project

# File -> Build Settings -> Android -> Build and Run

# Manual fallback: adb install -r Builds/HoloAssist.apk

# On Quest: App Library -> Unknown Sources -> HoloAssist

5.  Subsystems

Template

5.1  Perception

Owner: John Chen    |    Tech stack: Python 3.10 / ROS 2 Humble / apriltag_ros / Intel RealSense SDK 2.x

Package(s): ros2_ws/src/holo_assist_depth_tracker/    ros2_ws/src/holo_assist_depth_tracker_sim/

5.1.1  Walkthrough Script (approx. 5 minutes)

Tick each step as you cover it. Sections below are in the same order — scroll down as you present.

Walkthrough Script — follow this with your coach

Step 1. Orient your coach: 'Perception detects the four cubes and workspace board using AprilTag markers on a RealSense D435i camera, and publishes metric poses so the Sorter can pick them and the XR scene can display them.'

Step 2. Show Purpose & Design Rationale — explain why AprilTags were chosen over depth-based clustering.

Step 3. Show How It Works — two-stage pipeline: workspace board lock (SVD/Kabsch), then per-cube multi-face consensus.

Step 4. Open holoassist_workspace_board_node.py and holoassist_cube_pose_node.py — point to the SVD/Kabsch call and the face-consensus averaging loop.

Step 5. Show Inputs/Outputs — emphasise cube poses are in workspace_frame, not base_link.

Step 6. Run the simulation twin: ros2 launch holo_assist_depth_tracker sim_holoassist_trolley.launch.py gui:=true — narrate topic output.

Step 7. Open Parameter Reference — explain apriltag_size: mismatch causes proportional metric drift.

Step 8. Walk through one Troubleshooting row with reasoning.

Step 9. Close with Limitations — one-time workspace lock, exact tag size requirement.

Step 10. Invite questions.

5.1.2  Purpose & Design Rationale

Purpose:

Detects the four sortable cubes and the shared workspace board in real time using AprilTag fiducial markers on a ceiling-mounted Intel RealSense D435i. Publishes metric 3-D poses for each cube in a stable workspace_frame so Autonomous Sorting can plan pick-and-place motions and XR Scene can overlay status information at the correct physical locations.

Role in the integrated system:

Subscribes to raw colour and depth images from the RealSense driver. Publishes per-cube poses consumed by Autonomous Sorting (via the pose-handoff adapter) and colour-coded RViz markers for debugging.

Key design decisions and why:

- AprilTag markers instead of depth-based clustering — tags give metric 6-DOF pose in a single frame with no hyperparameter tuning; clustering requires careful RANSAC+DBSCAN thresholds and breaks down for similarly-shaped objects.

- Two-stage pipeline (board lock first, then cube poses) — workspace_frame from the four corner tags (IDs 0-3) provides a stable coordinate origin that survives minor camera movement without recalibrating the full TF chain.

- Multi-face consensus per cube (6 face tags, IDs 10-33) — averaging visible face measurements reduces pose variance from partial occlusion; a single visible face is sufficient but noisier.

- Simulation twin (depth_tracker_sim) publishes on the same topics as the real node — downstream subsystems are unaffected and can be developed without the physical camera.

5.1.3  How It Works — Internal Logic

Point to the implementing code for each step during the walkthrough.

Processing pipeline:

- RealSense driver publishes /camera/camera/color/image_raw and /camera/camera/depth/image_rect_raw at 30 Hz.

- apriltag_ros detects all visible AprilTag IDs and publishes raw detections on /detections_all.

- holoassist_workspace_board_node.py reads tags 0-3 (board corners), computes workspace_frame via SVD/Kabsch alignment, and broadcasts a static TF. Once locked, the frame does not update until /holoassist/perception/realign_workspace is called.

- holoassist_cube_pose_node.py reads tags 10-33 (6 face tags per cube, 4 cubes). For each visible face tag, it computes the cube centre by applying a 20 mm face-to-centre offset. Visible face results are averaged (consensus). Zero faces = 'not_detected'.

- Cube centres published as geometry_msgs/PoseStamped in workspace_frame on /holoassist/perception/april_cube_N_pose at ~10 Hz when detected.

- Per-cube status published on /holoassist/perception/april_cube_N_status.

- Colour-coded RViz markers published on /holoassist/perception/april_cube_N_marker (1=red, 2=green, 3=blue, 4=orange).

TIP Open the source file and point to the lines implementing each step above.

Data flow:

[Insert: RealSense colour/depth -> apriltag_ros /detections_all -> workspace_board_node (SVD) -> workspace_frame TF; detections -> cube_pose_node (face consensus) -> /holoassist/perception/april_cube_N_pose in workspace_frame -> Sorting adapter + RViz]

5.1.4  Key Files & Code Structure

| File / Node | Path | Role | Key Lines to Show |
| --- | --- | --- | --- |
| holoassist_workspace_board_node.py | ros2_ws/src/holo_assist_depth_tracker/holo_assist_depth_tracker/ | Solves and locks workspace_frame from corner tags 0-3 (SVD/Kabsch) | [Fill in key line ranges] |
| holoassist_cube_pose_node.py | ros2_ws/src/holo_assist_depth_tracker/holo_assist_depth_tracker/ | Multi-face consensus pose per cube (IDs 10-33) | [Fill in key line ranges] |
| holo_assist_depth_tracker_node.py | ros2_ws/src/holo_assist_depth_tracker/holo_assist_depth_tracker/ | Depth integration and debug overlay publisher | [Fill in key line ranges] |
| holoassist_4tag_board_4cube.launch.py | ros2_ws/src/holo_assist_depth_tracker/launch/ | Main hardware launch: RealSense + apriltag_ros + both nodes | All — open whole file |
| sim_holoassist_trolley.launch.py | ros2_ws/src/holo_assist_depth_tracker/launch/ | Simulation: synthetic tag publisher + Gazebo | All — open whole file |
| config/apriltag_all.yaml | ros2_ws/src/holo_assist_depth_tracker/config/ | All 34 tag IDs, physical size 0.032 m | All — open whole file |
| config/workspace.yaml | ros2_ws/src/holo_assist_depth_tracker/config/ | Board geometry: 0.7 m x 0.5 m, tag size | All — open whole file |
| config/cubes.yaml | ros2_ws/src/holo_assist_depth_tracker/config/ | Cube ID groups and face-to-centre offset mapping | All — open whole file |

5.1.5  Inputs & Outputs

Inputs:

| Input | Source | Data Type / Format | Notes |
| --- | --- | --- | --- |
| /camera/camera/color/image_raw | RealSense driver | sensor_msgs/Image | 30 Hz — consumed by apriltag_ros |
| /camera/camera/depth/image_rect_raw | RealSense driver | sensor_msgs/Image | 30 Hz — depth integration |
| /camera/camera/color/camera_info | RealSense driver | sensor_msgs/CameraInfo | Camera intrinsics for tag pose computation |
| /detections_all | apriltag_ros | apriltag_msgs/AprilTagDetectionArray | All visible tag poses consumed by both board and cube nodes |

Outputs:

| Output | Destination | Data Type / Format | Notes |
| --- | --- | --- | --- |
| /holoassist/perception/april_cube_N_pose (N=1-4) | Autonomous Sorting adapter | geometry_msgs/PoseStamped | Cube centre in workspace_frame, ~10 Hz |
| /holoassist/perception/april_cube_N_status (N=1-4) | Diagnostics / XR Scene | std_msgs/String | 'detected' or 'not_detected' per cube |
| /holoassist/perception/april_cube_N_marker (N=1-4) | RViz | visualization_msgs/Marker | Colour-coded markers |
| /holoassist/perception/workspace_mode | All subsystems | std_msgs/String | 'locked' / 'searching' |
| TF: workspace_frame | All subsystems | tf2_msgs/TFMessage | Stable board-origin frame (static once locked) |

5.1.6  ROS Interfaces

Published topics:

| Topic | Msg Type | Rate | Description |
| --- | --- | --- | --- |
| /holoassist/perception/april_cube_N_pose | geometry_msgs/PoseStamped | ~10 Hz | Cube centre in workspace_frame |
| /holoassist/perception/april_cube_N_status | std_msgs/String | ~10 Hz | Detection state per cube |
| /holoassist/perception/april_cube_N_marker | visualization_msgs/Marker | ~10 Hz | RViz colour-coded marker |
| /holoassist/perception/workspace_mode | std_msgs/String | On change | 'locked' or 'searching' |

Subscribed topics:

| Topic | Msg Type | Description / How Used |
| --- | --- | --- |
| /detections_all | apriltag_msgs/AprilTagDetectionArray | Raw AprilTag detections — consumed by both nodes |
| /camera/camera/color/camera_info | sensor_msgs/CameraInfo | Camera intrinsics for metric pose |

Services / Actions:

| Name | Type | Request -> Response | Description |
| --- | --- | --- | --- |
| /holoassist/perception/realign_workspace | std_srvs/Trigger | [empty] -> success: bool | Re-solves workspace_frame if the board moves |

5.1.7  Running & Testing Independently

Prerequisites:

- RealSense D435i plugged into USB 3.0 (blue port).

- AprilTags printed at exactly 32 mm edge size on board corners and cube faces.

- source /opt/ros/humble/setup.bash && source ros2_ws/install/setup.bash

Launch commands:

# Hardware

source /opt/ros/humble/setup.bash && source ros2_ws/install/setup.bash

ros2 launch holo_assist_depth_tracker holoassist_4tag_board_4cube.launch.py

# Simulation (no camera needed)

ros2 launch holo_assist_depth_tracker sim_holoassist_trolley.launch.py gui:=true

Test input:

ros2 topic echo /holoassist/perception/workspace_mode --once

ros2 topic echo /holoassist/perception/april_cube_1_pose --once

Verify output:

ros2 topic hz /holoassist/perception/april_cube_1_pose  # expect ~10 Hz when cube 1 visible

EXPECTED workspace_mode reports 'locked' within a few seconds. Detected cube topics publish at ~10 Hz. In RViz, colour-coded markers appear at the correct physical positions relative to the board.

[Insert screenshot of expected terminal output / headset view here]

5.1.8  Parameter Reference

Coaches often ask 'what happens if I change X?'. The two right columns are the HD differentiator.

| Parameter | Location | Default | Valid Range | Effect of Increasing | What Breaks if Wrong |
| --- | --- | --- | --- | --- | --- |
| apriltag_size | config/apriltag_all.yaml | 0.032 m | Must match physical print | N/A — physical property | Any mismatch causes proportional metric drift: 10% size error = 10% pose error |
| workspace board dims (0.7 x 0.5 m) | config/workspace.yaml | 0.700 / 0.500 m | Must match physical board | N/A — physical property | Wrong dims skew the SVD alignment; board corner tags won't align in workspace_frame |
| cube_edge_size | config/cubes.yaml | 0.040 m | Must match physical cube | N/A — physical property | Wrong size offsets cube-centre estimate by half the error (face-to-centre = cube_edge / 2) |
| face_offset | config/cubes.yaml | 0.020 m | cube_edge_size / 2 | N/A — derived | Incorrect offset shifts all cube-centre estimates; reduces grasp accuracy |
| [Add apriltag_ros detector params...] | config/apriltag_all.yaml |  |  |  |  |

TIP Prepare to demonstrate one live parameter change — edit a value, show the behaviour difference.

5.1.9  Live Configuration Changes

Scenario — "Workspace moved — re-lock workspace_frame":

- Call: ros2 service call /holoassist/perception/realign_workspace std_srvs/Trigger

- Reasoning: the board node re-reads all four corner tag detections and re-runs SVD/Kabsch; workspace_frame shifts to match the board's new physical position.

- Verify: ros2 topic echo /holoassist/perception/workspace_mode briefly reports 'searching' then 'locked'.

Scenario — "Switch to simulation (no camera / cubes needed)":

- Stop the hardware launch if running.

- ros2 launch holo_assist_depth_tracker sim_holoassist_trolley.launch.py gui:=true

- Reasoning: the sim twin publishes synthetic poses on the same topics; Sorting and XR Scene are unaffected.

5.1.10  Troubleshooting (with Reasoning)

The 'Why the Fix Works' column separates HD from Credit.

| Symptom | Likely Cause & Why | How to Diagnose | Fix | Why the Fix Works |
| --- | --- | --- | --- | --- |
| workspace_mode stays 'searching' | Board corner tags 0-3 not all visible — SVD requires all four | ros2 topic echo /detections_all — check which IDs appear | Move camera until all four board corners are in frame | SVD/Kabsch alignment needs four non-collinear anchor points; fewer than four leaves the system under-determined |
| Cube pose drifts or jumps | apriltag_size in config does not match the physical printed size | Measure tag edge with calipers; compare to config/apriltag_all.yaml | Update apriltag_size to the measured value and re-launch | Tag pose is computed using declared size as metric scale; any error is a direct proportional offset in X/Y/Z |
| Cube reported 'not_detected' when visible | Visible face tag ID not in the expected range for that cube | ros2 topic echo /detections_all and check IDs | Confirm face labels on physical cube match config/cubes.yaml ID groups | Each cube's ID group must exactly match printed tags; a mislabelled face never contributes to consensus |
| No output after hardware launch | RealSense not found on USB | ros2 run realsense2_camera realsense2_camera_node — look for camera serial | Replug into USB 3.0 (blue); check udev rules for RealSense | RealSense needs USB 3.0 bandwidth for simultaneous colour+depth; USB 2.0 silently prevents driver start |
| [Add more as discovered...] |  |  |  |  |

5.1.11  Limitations & Assumptions

Assumptions:

- AprilTags printed at exactly 32 mm edge — any deviation causes proportional metric drift.

- Camera rigidly mounted with a calibrated static TF to base_link (from easy_handeye2; see CALIBRATION.md).

- All four board corner tags (IDs 0-3) visible simultaneously during initialisation.

- Board does not move after workspace_frame is locked (or realign_workspace called after each move).

Known limitations:

- Workspace frame solved once and locked — moving board or camera requires manual realign_workspace call.

- Simulation twin uses pre-placed deterministic cube positions — does not simulate camera noise or occlusion.

- Partially occluded cubes (zero visible face tags) reported as 'not_detected', not estimated.

- No object classification — tracks cubes by AprilTag ID only.

Future work:

- Continuous workspace_frame updates with outlier rejection for robustness to occasional tag dropouts.

- Depth-fused pose refinement to improve metric accuracy beyond tag pose alone.

- Support more than 4 cubes by extending ID groups in config/cubes.yaml.

5.1.12  Anticipated Coach Questions

Pre-prepare answers so you respond confidently.

Q1:  Why AprilTags instead of point-cloud clustering?

A1:  AprilTags give a metric 6-DOF pose from a single image frame with no training data and no hyperparameter tuning. Point-cloud clustering requires careful threshold tuning per object shape and breaks down with similarly-sized adjacent objects. For known objects with printed tags, fiducial detection is faster, more reliable, and easier to debug.

Q2:  What happens if one cube face tag is damaged?

A2:  The consensus averages over all visible face tags. With 5 of 6 faces intact the estimate is unaffected. If only 1 face is visible the pose is noisier but still published. If 0 faces are visible the cube is reported 'not_detected' until a face comes into view.

Q3:  How is workspace_frame related to base_link?

A3:  workspace_frame is the board-corner origin published by the board node. The pose-handoff adapter looks up the TF from workspace_frame to base_link to transform cube poses into the MoveIt planning frame. The calibration transform is provided by easy_handeye2 (see CALIBRATION.md).

Q4:  What is the latency from cube movement to published pose?

A4:  apriltag_ros runs at the camera frame rate (30 Hz). The cube pose node processes each detection batch and publishes at ~10 Hz. End-to-end latency from physical movement to published pose is typically 30-100 ms depending on ROS scheduling.

Q5:  [Add your own anticipated question]

A5:  [Your answer]

5.2  Autonomous Sorting

Owner: Oliver Lau    |    Tech stack: Python / C++ / ROS 2 Humble / MoveIt 2 / ur_onrobot_control

Package(s): ros2_ws/src/moveit_robot_control/    ros2_ws/src/moveit_robot_control_msgs/

5.2.1  Walkthrough Script (approx. 5 minutes)

Tick each step as you cover it. Sections below are in the same order — scroll down as you present.

Walkthrough Script — follow this with your coach

Step 1. Orient your coach: 'Autonomous Sorting receives cube poses from Perception and plans + executes collision-free pick-and-place on the UR3e via MoveIt 2 — no operator input needed.'

Step 2. Show Purpose & Design Rationale — explain Cartesian-first / pose-goal fallback and why a state machine was used instead of MoveIt Task Constructor.

Step 3. Show How It Works — walk through the state machine: pregrasp -> grasp -> lift -> bin -> release -> return.

Step 4. Open pick_place_sequencer.py and coordinate_listener.py — point to the state machine transitions and MoveIt planning calls.

Step 5. Show Inputs/Outputs — emphasise /holoassist/pick_cube_to_bin as the entry point.

Step 6. Demonstrate: ros2 service call /holoassist/pick_cube_to_bin ... '{cube_name: april_cube_1, bin_id: bin_1}'

Step 7. Open Parameter Reference — explain hover_offset and planning_time as most commonly tuned.

Step 8. Walk through one Troubleshooting row with reasoning.

Step 9. Close with Limitations — sequential only, no regrasp, conservative collision avoidance.

Step 10. Invite questions.

5.2.2  Purpose & Design Rationale

Purpose:

Receives detected cube poses from Perception and plans and executes collision-free pick-and-place motions using MoveIt 2, the UR3e arm, and the OnRobot RG2 gripper. Operates fully autonomously without operator input, triggered by a single service call naming the cube and destination bin.

Role in the integrated system:

Subscribes to cube poses from Perception via the pose-handoff adapter (which transforms poses from workspace_frame to base_link). Publishes joint trajectories to the ur_onrobot_control driver and gripper commands to /finger_width_controller/commands. Maintains MoveIt planning scene collision objects. Reports progress on /moveit_robot_control/state.

Key design decisions and why:

- Cartesian path first, OMPL pose-goal fallback — straight-line Cartesian approach is predictable for the operator but can fail near singularities; the OMPL fallback finds any collision-free path, ensuring robustness.

- State machine in pick_place_sequencer.py rather than MoveIt Task Constructor — simpler to debug and modify without recompiling; each state (pregrasp, grasp, lift, place, release, retreat) is explicit and individually testable.

- workspace_scene_manager.py adds collision geometry (trolley mesh, bin walls, workspace bounds) to MoveIt — prevents the planner routing through physical obstacles even if the operator does not see them.

- Service-based entry point rather than topic — a service blocks until completion and returns success/failure, giving the caller explicit feedback; a topic would fire-and-forget.

5.2.3  How It Works — Internal Logic

Point to the implementing code for each step during the walkthrough.

Processing pipeline:

- Caller triggers /holoassist/pick_cube_to_bin with cube_name and bin_id.

- coordinate_listener.py looks up the latest pose for cube_name on /moveit_robot_control/target_pose (in base_link, from the adapter).

- State machine: PREGRASP — plans Cartesian path to hover_offset above the cube. Executes.

- GRASP — plans Cartesian descent to grasp pose. Opens RG2 before descent; closes at the bottom.

- LIFT — Cartesian retreat to hover height. Attaches cube as collision child of gripper in the planning scene.

- MOVE_TO_BIN — joint-space plan to above the target bin (from bin_poses.json). Cartesian descent. Opens RG2.

- RETREAT — Cartesian path to safe home. Reports COMPLETE or FAILED on /moveit_robot_control/state.

- workspace_scene_manager updates collision objects throughout (removes cube from scene on pick, adds to bin on place).

TIP Open the source file and point to the lines implementing each step above.

Data flow:

[Insert: /holoassist/pick_cube_to_bin -> coordinate_listener.py gets pose -> pick_place_sequencer.py state machine -> MoveIt 2 Cartesian/joint plan -> ur_onrobot_control (/forward_velocity_controller); gripper: /finger_width_controller/commands; status: /moveit_robot_control/state]

5.2.4  Key Files & Code Structure

| File / Node | Path | Role | Key Lines to Show |
| --- | --- | --- | --- |
| coordinate_listener.py | ros2_ws/src/moveit_robot_control/moveit_robot_control/ | Subscribes to target_pose; calls MoveIt Cartesian first, pose fallback | [Fill in key line ranges] |
| pick_place_sequencer.py | ros2_ws/src/moveit_robot_control/moveit_robot_control/ | State machine: pregrasp->grasp->lift->bin->release->return | [Fill in key line ranges] |
| workspace_scene_manager.py | ros2_ws/src/moveit_robot_control/moveit_robot_control/ | Manages MoveIt collision objects (trolley, bins, workspace bounds) | [Fill in key line ranges] |
| full_holoassist_hardware.launch.py | ros2_ws/src/moveit_robot_control/launch/ | Full hardware stack: UR driver + MoveIt + perception + sequencer | All — open whole file |
| full_holoassist_moveit_sim.launch.py | ros2_ws/src/moveit_robot_control/launch/ | Full simulation: fake hardware + MoveIt + perception sim + sequencer | All — open whole file |
| config/bin_poses.json | ros2_ws/src/moveit_robot_control/config/ | Target gripper poses for each bin (world coordinates) | All — open whole file |
| config/full_holoassist_hw.yaml | ros2_ws/src/moveit_robot_control/config/ | Hardware-mode params (workspace TF source, adapter offsets) | All — open whole file |

5.2.5  Inputs & Outputs

Inputs:

| Input | Source | Data Type / Format | Notes |
| --- | --- | --- | --- |
| Pick command | Service caller | holo_assist_depth_tracker_sim_interfaces/srv/PickCubeToBin | cube_name + bin_id; blocks until done |
| Cube pose (base_link) | Perception adapter (/moveit_robot_control/target_pose) | geometry_msgs/PoseStamped | Transformed from workspace_frame by the adapter |
| Robot joint states | /joint_states from ur_onrobot_control | sensor_msgs/JointState | Required by MoveIt for current-state planning |
| TF: workspace_frame -> base_link | easy_handeye2 calibration publisher | tf2_msgs/TFMessage | Allows adapter to transform cube poses into planning frame |

Outputs:

| Output | Destination | Data Type / Format | Notes |
| --- | --- | --- | --- |
| /moveit_robot_control/state | XR Scene + dashboard | std_msgs/String | QUEUED / PLANNING / PLANNED / EXECUTING / COMPLETE / FAILED |
| /moveit_robot_control/status | Diagnostics | std_msgs/String | Human-readable current action |
| /moveit_robot_control/debug | Diagnostics | std_msgs/String | JSON plan details (step, waypoints, timing) |
| /finger_width_controller/commands | OnRobot RG2 driver | trajectory_msgs/JointTrajectory | Gripper open (0.11 m) / close (0.0 m) |

5.2.6  ROS Interfaces

Published topics:

| Topic | Msg Type | Rate | Description |
| --- | --- | --- | --- |
| /moveit_robot_control/state | std_msgs/String | On change | State machine lifecycle per pick |
| /moveit_robot_control/status | std_msgs/String | On change | Human-readable current action |
| /moveit_robot_control/debug | std_msgs/String | On change | JSON plan diagnostics |

Subscribed topics:

| Topic | Msg Type | Description / How Used |
| --- | --- | --- |
| /moveit_robot_control/target_pose | geometry_msgs/PoseStamped | Goal pose in base_link — from the perception-to-motion adapter |
| /joint_states | sensor_msgs/JointState | Current robot state required for all MoveIt planning |

Services / Actions:

| Name | Type | Request -> Response | Description |
| --- | --- | --- | --- |
| /holoassist/pick_cube_to_bin | PickCubeToBin service | cube_name + bin_id -> success: bool | Blocks until full pick-and-place completes or fails |
| /sort/cancel | std_srvs/Trigger | [empty] -> success: bool | Aborts in-progress pick and stops robot safely |

5.2.7  Running & Testing Independently

Prerequisites:

- UR3e on, remote control mode, External Control program running (host IP 192.168.0.100).

- Full ROS stack: ./launch.sh --robot-ip 192.168.0.192 (or ./launch.sh for simulation).

- Perception subsystem running and publishing cube poses.

Launch commands:

# Simulation

source /opt/ros/humble/setup.bash && source ros2_ws/install/setup.bash

ros2 launch moveit_robot_control full_holoassist_moveit_sim.launch.py

# Hardware

ros2 launch moveit_robot_control full_holoassist_hardware.launch.py robot_ip:=192.168.0.192

Test input:

ros2 service call /holoassist/pick_cube_to_bin \

holo_assist_depth_tracker_sim_interfaces/srv/PickCubeToBin \

"{cube_name: 'april_cube_1', bin_id: 'bin_1'}"

Verify output:

ros2 topic echo /moveit_robot_control/state  # watch: QUEUED->PLANNING->PLANNED->EXECUTING->COMPLETE

EXPECTED State topic cycles QUEUED -> PLANNING -> PLANNED -> EXECUTING -> COMPLETE. On hardware the UR3e physically picks cube 1 and places it in bin_1. In simulation the robot model in RViz follows the planned trajectory.

[Insert screenshot of expected terminal output / headset view here]

5.2.8  Parameter Reference

Coaches often ask 'what happens if I change X?'. The two right columns are the HD differentiator.

| Parameter | Location | Default | Valid Range | Effect of Increasing | What Breaks if Wrong |
| --- | --- | --- | --- | --- | --- |
| hover_offset | config/bin_poses.json (per cube) | ~0.10 m | 0.05-0.20 m | More clearance before descent — safer but slower cycle | Too small -> arm clips object before grasp; too large -> wasted travel time |
| planning_time | config/full_holoassist_hw.yaml | 5.0 s | 1.0-30.0 s | More OMPL retries before failure | Too small -> spurious planning failures; too large -> long wait before reporting FAILED |
| max_velocity_scaling | full_holoassist_hardware.launch.py | 0.3 | 0.05-1.0 | Faster motions, shorter cycle time | Too high -> protective stop trigger; too low -> tediously slow |
| max_acceleration_scaling | full_holoassist_hardware.launch.py | 0.3 | 0.05-1.0 | Snappier start/stop | Too high -> safety stop; too low -> sluggish |
| gripper_close_width | pick_place_sequencer.py | 0.0 m (fully closed) | 0.0-0.085 m | Wider -> softer grip for delicate objects | Too narrow -> motor stall; too wide -> drops object mid-transfer |
| [Add bin_poses.json entries...] | config/bin_poses.json |  |  |  |  |

TIP Prepare to demonstrate one live parameter change — edit a value, show the behaviour difference.

5.2.9  Live Configuration Changes

Scenario — "Add a new drop-off bin":

- Open config/bin_poses.json.

- Add entry: 'bin_3': {'position': [x, y, z], 'orientation': [qx, qy, qz, qw]} in base_link frame.

- Reasoning: bin poses loaded at node startup by name string; no code change required.

- Re-launch and call: ros2 service call /holoassist/pick_cube_to_bin ... '{cube_name: april_cube_1, bin_id: bin_3}'

Scenario — "Slow down for a live demo":

- Set max_velocity_scaling and max_acceleration_scaling from 0.3 to 0.15 in the launch file.

- Reasoning: MoveIt scales the TOTG time-parameterisation uniformly — same path at half speed.

- Re-launch: ros2 launch moveit_robot_control full_holoassist_hardware.launch.py robot_ip:=192.168.0.192

Scenario — "Increase planning robustness in a cluttered scene":

- Raise planning_time from 5.0 to 10.0 s in config/full_holoassist_hw.yaml.

- Reasoning: OMPL is a randomised planner; more time = more samples = higher success probability.

- Trade-off: longer wait before a FAILED result is returned to the caller.

5.2.10  Troubleshooting (with Reasoning)

The 'Why the Fix Works' column separates HD from Credit.

| Symptom | Likely Cause & Why | How to Diagnose | Fix | Why the Fix Works |
| --- | --- | --- | --- | --- |
| Service returns success: false immediately | Cube pose not available — Perception not running or cube not detected | ros2 topic echo /moveit_robot_control/target_pose — if no messages, Perception is not publishing | Start the Perception subsystem or sim twin; ensure cube is visible to the camera | The sequencer reads the latest available pose; if none exists it cannot form a valid goal and returns false immediately |
| State stalls at PLANNING beyond planning_time | MoveIt cannot find a collision-free path — goal in collision or scene too cluttered | Open RViz, inspect PlanningScene for unexpected collision geometry at the goal pose | Remove or reposition the blocking object; call workspace_scene_manager to refresh scene | OMPL samples random configurations; if the goal state itself is in collision no plan can succeed regardless of time budget |
| Robot stops mid-motion (protective stop) | Velocity/acceleration scaling too high for current payload + trajectory | Check teach pendant: 'Protective stop' | Lower max_velocity_scaling to 0.2 and max_acceleration_scaling to 0.2; re-launch | UR safety controller enforces joint torque limits; smoother acceleration keeps forces below the threshold |
| Gripper closes but cube falls mid-lift | Grasp pose too high — gripper closed in air above cube surface | Watch gripper position in RViz during descent; if it goes fully closed before reaching the object the descent ended early | Reduce hover_offset for that cube; or check if workspace_frame->base_link TF calibration is accurate | Grasp depth = object_pose + hover_offset; if Perception reports pose too high the gripper never reaches the cube surface |
| [Add more as discovered...] |  |  |  |  |

5.2.11  Limitations & Assumptions

Assumptions:

- UR3e URDF accurately reflects physical configuration including RG2 mass and offset.

- All cubes fit within RG2 jaw range (0-85 mm).

- workspace_frame locked and accurate before any sort is triggered — stale workspace_frame = wrong grasp targets.

- MoveIt planning scene is current; unexpected physical obstacles not listed are not avoided.

Known limitations:

- Sequential sorting only — one cube at a time; no batch or parallel planning.

- No regrasp on slip — if cube slips mid-transfer the cycle fails and must be re-triggered.

- Collision avoidance is conservative — may reject feasible paths when bins are close to workspace boundaries.

- Simulation perception is deterministic — camera noise and occlusion are not modelled.

Future work:

- MoveIt Task Constructor for multi-object batch planning.

- Force-feedback from RG2 to detect and recover from grasp slips.

- Adaptive hover offset based on cube height estimate from Perception.

5.2.12  Anticipated Coach Questions

Pre-prepare answers so you respond confidently.

Q1:  Why Cartesian path first, OMPL pose-goal as fallback?

A1:  Cartesian paths are straight-line approaches — predictable, visually clear for the operator, and interpretable in MoveIt. However they can fail if a straight-line requires passing through a singular configuration or a collision. The OMPL fallback finds any collision-free path, ensuring the pick succeeds even when the ideal approach is blocked.

Q2:  How does the planning scene stay consistent with reality?

A2:  workspace_scene_manager.py adds collision geometry for the trolley and bins at startup, removes the cube from the scene when picked, and adds it to the bin when placed. If a new physical obstacle appears it must be explicitly added — there is no real-time obstacle detection.

Q3:  What is the Perception-to-Sorting handoff contract?

A3:  Perception publishes cube poses in workspace_frame. The adapter (selected_cube_to_moveit_target_node.py) does a TF lookup (workspace_frame -> base_link) and republishes on /moveit_robot_control/target_pose in base_link. The full contract is in docs/POSE_HANDOFF_CONTRACT.md.

Q4:  What if the pick service is called while the robot is already executing?

A4:  The service blocks until the current pick completes or fails, then runs the new pick. To abort an in-progress pick, call /sort/cancel first.

Q5:  [Add your own anticipated question]

A5:  [Your answer]

5.3  Teleoperation & Interaction

Owner: Nicholas Sabatta (Nic)    |    Tech stack: C# (Unity 6.3 LTS) / Python (ROS 2 Humble) / PyQt5

Package(s): Unity/My project/Assets/Scripts/    ros2_ws/src/ROS-TCP-Endpoint/    dashboard/

5.3.1  Subsystem Overview

This section provides a high-level technical summary of the Teleoperation & Interaction subsystem. Detailed specifications follow in subsequent sections.

Subsystem Summary — Teleoperation & Interaction

Step 1. The Teleoperation & Interaction subsystem enables real-time operator control of the UR3e arm via three input modes: RMRC (Resolved Motion Rate Control using Jacobian-based velocity commands), Direct Joint jogging, and Hand Guide tracking. It also provides an E-stop safety dashboard, session logging, and a PyQt5 e-stop dashboard for remote monitoring.

Step 2. Three control modes serve distinct operational needs. RMRC provides intuitive Cartesian control via damped least-squares (DLS) Jacobian inversion, which prevents velocity divergence near singularities. Direct Joint jogging allows recovery from singular configurations. Hand Guide enables trajectory demonstration via controller tracking.

Step 3. Control loop architecture: Meta XR SDK controller input (50 Hz) → active mode handler → Jacobian DLS inverse (for RMRC) → exponential smoothing (τ = 0.06 s) → 50 Hz joint velocity commands published to /forward_velocity_controller/commands via ros_tcp_endpoint.

Step 4. Core implementation files: RobotController.cs (mode dispatch, input mapping, 50 Hz publish loop, output smoothing) and UR3eKinematics.cs (forward kinematics, 6×6 geometric Jacobian computation, DLS pseudoinverse).

Step 5. Controller mapping: right thumbstick XY → Cartesian XY velocity; left thumbstick Y → Cartesian Z velocity; right trigger (analog) → gripper width (0–0.11 m); left Menu button → mode cycle; A/B buttons → joint selection (Direct Joint mode).

Step 6. Key tunable parameters: linearSpeed (default 0.25 m/s, requires field tuning), jointJogSpeed (default 0.5 rad/s), outputSmoothTime (0.06 s exponential low-pass time constant), handGuideLateralGain (2.0), handGuideLateralCapM (0.15 m/s).

Step 7. Known critical issue: ROS-TCP-Connector v0.7.0 race condition causes subscriber registrations to be lost after Unity Play/Stop cycles. Workaround: restart ros_tcp_endpoint before re-entering Play mode.

Step 8. Current limitations: end-to-end WiFi latency of 30–80 ms, MeshCollisionGuard disabled pending calibration, linearSpeed and jointJogSpeed values require tuning on physical hardware.

Step 9. Refer to sections 5.3.10 (Troubleshooting) and 5.3.12 (FAQ) for common issues and solutions.

5.3.2  Purpose & Design Rationale

Purpose:

Enables the operator wearing a Meta Quest 3 to directly control the UR3e arm and RG2 gripper in real time across three input modes: RMRC (Resolved Motion Rate Control), Direct Joint jogging, and Hand Guide tracking. Provides a PyQt5 E-stop dashboard, and per-session metrics logging.

Role in the integrated system:

Reads Meta XR SDK controller/hand inputs, computes joint velocity commands, and sends them to the UR3e driver via ros_tcp_endpoint. Subscribes to /joint_states for digital-twin feedback. The E-stop halts motion via burst-zero publishing and controller deactivation.

Key design decisions and why:

- Three modes (RMRC / Direct Joint / Hand Guide) each is optimal for different tasks: RMRC for precise Cartesian approach, Direct Joint for recovering from singular configurations, Hand Guide for trajectory demonstration.

- Damped least-squares (DLS) Jacobian inversion in RMRC — standard Jacobian inversion diverges near singularities; DLS trades a small precision loss for bounded output.

- ros_tcp_endpoint (TCP, port 10000) instead of rosbridge WebSocket — lower and more predictable latency; rosbridge adds JSON serialisation overhead inappropriate for 50 Hz commands.

- E-stop burst-publishes zeros (10x) before switching controllers — ensures stop reaches the robot even with a dropped packet; continuous zero publishing maintains the stop state.

- Session logging runs as a background coroutine so it does not block the 50 Hz control loop.

5.3.3  How It Works — Internal Logic

The following describes the internal processing pipeline and data flow.

Processing pipeline:

- Unity polls Meta XR SDK at 72 Hz for controller pose, stick axes, and button state.

- ROSAutoConnect.cs TCP-scans 192.168.0.101-109:10000 to find ros_tcp_endpoint; connects automatically.

- Active mode determines command generation: RMRC Translate (stick XY + left stick Y -> Cartesian velocity -> DLS Jacobian inverse -> 6 joint velocities); RMRC Rotate (sticks -> wrist joint velocities directly); Direct Joint (thumbstick Y -> proportional velocity for selected joint); Hand Guide (right grip held + controller 3D delta -> Jacobian IK, capped at 0.15 m/s).

- Right trigger (analog) maps 0-100% to /finger_width_controller/commands (0 = closed, 0.11 m = open).

- outputSmoothTime (0.06 s exponential low-pass) applied to all velocities before publishing at 50 Hz to /forward_velocity_controller/commands.

- SessionLogger.cs records mode switches and durations; publishes JSON on /session/status at 2 Hz; saves to file on quit.

NOTE See Section 5.3.4 (Key Files & Code Structure) for file paths and relevant line ranges.

Data flow:

[Insert: Quest controller/hand -> Unity Input System -> mode handler -> UR3eKinematics.cs DLS Jacobian -> /forward_velocity_controller/commands -> ros_tcp_endpoint -> ROS -> UR3e. Feedback: /joint_states -> JointStateSubscriber -> digital twin + HUD. E-stop: dashboard button -> burst zeros -> controller switch]

5.3.4  Key Files & Code Structure

| File / Node | Path | Role | Key Lines to Show |
| --- | --- | --- | --- |
| RobotController.cs | Unity/My project/Assets/Scripts/ | Three control modes, gripper mapping, 50 Hz publish, output smoothing | L15-52 (params), L150 (XRI disable), L251 (Update loop), L391-543 (velocity/gripper publish) — 678 lines total |
| JointStateSubscriber.cs | Unity/My project/Assets/Scripts/ | Subscribes to /joint_states; drives robot joint transforms + caches LastJointAnglesRad[] | L13-35 (rosToUnity map), L52 (Start), L75 (OnJointState callback) — 118 lines total |
| UR3eKinematics.cs | Unity/My project/Assets/Scripts/ | Forward kinematics, geometric Jacobian, DLS pseudoinverse | L4-115 (DH params + FK), L117 (ResolveLinearVelocity), L231 (ResolveAngularVelocity), L267 (ResolveFullVelocity), L395 (GetToolZAxis) — 430 lines total |
| MeshCollisionGuard.cs | Unity/My project/Assets/Scripts/ | Predictive collision protection (table + self-collision) — disabled pending tuning | L89 (ComputeVelocityScale), table + self-collision logic — 185 lines total |
| SessionLogger.cs | Unity/My project/Assets/Scripts/ | Mode duration logging, E-stop count, /session/status at 2 Hz | L105 (PublishStatus), L255 (OnApplicationQuit), L271 (SaveSessionJSON) — 333 lines total |
| ROSAutoConnect.cs | Unity/My project/Assets/Scripts/ | TCP scan localhost → 192.168.0.101-110 → local IPs → fallback for auto-discovery of ros_tcp_endpoint | L41-76 (ScanForEndpoint: localhost → 192.168.0.101-110 → local IPs → fallback) — 96 lines total |
| HeadsetStreamPublisher.cs | Unity/My project/Assets/Scripts/ | XR camera view -> JPEG -> /headset/image_compressed at 15 FPS | All — 152 lines total |
| launch.py | root/ | Orchestrates: UR driver, controller switch (8 s wait), ros_tcp_endpoint, beacon | L103-295 (main): phases 1-4, --perception flag starts camera+apriltag+cube_pose_node+relay — 295 lines total |
| dashboard/main.py | root/dashboard/ | PyQt5 E-stop dashboard: 6 tabs, status bar, E-stop + RESUME controls | L91 (StatusBar), L393 (StatusScreen), L546 (HeadsetScreen), L607 (RollingGraph), L727 (StatsScreen), L838 (LatencyScreen), L951 (SessionScreen) — 1367 lines total |
| CubePoseSubscriber.cs | Unity/My project/Assets/Scripts/ | Subscribes to /holoassist/unity/cube_{1-4}_pose; spawns coloured cubes at detected positions relative to robotBase | L49 (Start/subscribe), L113 (OnCubePose: ROS→Unity coord transform), L149 (CreateVirtualObject) — 198 lines total |
| cube_pose_relay.py | ros2_ws/ | Relays cube poses from perception topics to Unity-friendly /holoassist/unity/cube_{N}_pose topics | All — 127 lines total |
| calibrate.py | root/ | Single-command calibration launcher: UR driver + RealSense + AprilTag + easy_handeye2 GUI (eye-on-base, tracking_base_frame:=camera_link) | All — 185 lines total |
| calibrate.sh | root/ | Shell wrapper: sources both nic + main ROS workspaces, then runs calibrate.py | All — short wrapper |

5.3.5  Inputs & Outputs

Inputs:

| Input | Source | Data Type / Format | Notes |
| --- | --- | --- | --- |
| Controller pose + buttons | Meta XR SDK (Quest 3) | Vector3 + Quaternion + analog axes | 72 Hz — see controller mapping table below |
| Controller position (Hand Guide) | Meta Quest 3 right controller | Vector3 position + grip trigger | Right grip held activates position tracking via Jacobian IK |
| /joint_states | ur_onrobot_control via ros_tcp_endpoint | sensor_msgs/JointState | 6 joints + finger_width; used for Jacobian + HUD display |
| /holoassist/unity/cube_{1-4}_pose | cube_pose_relay.py (perception pipeline) | geometry_msgs/PoseStamped | Cube positions in base_link frame; CubePoseSubscriber spawns coloured cubes in Unity |

Outputs:

| Output | Destination | Data Type / Format | Notes |
| --- | --- | --- | --- |
| /forward_velocity_controller/commands | ur_onrobot_control | std_msgs/Float64MultiArray | 6 joint velocities (rad/s) at 50 Hz |
| /finger_width_controller/commands | OnRobot RG2 driver | std_msgs/Float64MultiArray | Gripper width 0.0 (closed) to 0.11 m (open); proportional to right trigger |
| /session/status | Dashboard + logging | std_msgs/String | JSON: mode, per-mode durations, E-stop count at 2 Hz |
| /headset/image_compressed | Dashboard HEADSET tab | sensor_msgs/CompressedImage | XR camera JPEG at 15 FPS |
| /session/events | Dashboard + logging | std_msgs/String | Discrete JSON event on mode switch, e-stop, etc. |

5.3.6  ROS Interfaces

Published topics:

| Topic | Msg Type | Rate | Description |
| --- | --- | --- | --- |
| /forward_velocity_controller/commands | std_msgs/Float64MultiArray | 50 Hz | 6-DOF joint velocity command |
| /finger_width_controller/commands | std_msgs/Float64MultiArray | 50 Hz | Gripper position (0-0.11 m) from right trigger |
| /session/status | std_msgs/String | 2 Hz | JSON session metrics |
| /headset/image_compressed | sensor_msgs/CompressedImage | 15 Hz | Operator XR camera view |
| /session/events | std_msgs/String | On event | Discrete JSON event (mode switch, e-stop, etc.) |

Subscribed topics:

| Topic | Msg Type | Description / How Used |
| --- | --- | --- |
| /joint_states | sensor_msgs/JointState | Current robot state — drives digital twin + Jacobian computation |
| /holoassist/unity/cube_{1-4}_pose | geometry_msgs/PoseStamped | Cube positions from perception pipeline; CubePoseSubscriber spawns coloured cubes in Unity relative to robotBase |

Services / Actions:

| Name | Type | Request -> Response | Description |
| --- | --- | --- | --- |
| N/A — no services exposed | — | — | Mode switching is via Quest controller (Menu button), not a ROS service |

5.3.7  Running & Testing Independently

Prerequisites:

- ./launch.sh (fake hardware) or ./launch.sh --robot-ip 192.168.0.192 (real robot) — provides ros_tcp_endpoint on port 10000.

- Quest 3 in Developer Mode; connected to robot router (192.168.0.x subnet).

- Unity 6.3 LTS open; ROS IP set: Edit -> Project Settings -> ROS Settings -> IP = laptop Wi-Fi IP (e.g. 192.168.0.102), Port = 10000.

Launch commands:

# Terminal 1 — full ROS stack (fake hardware)

./launch.sh

# Terminal 1 — real robot

./launch.sh --robot-ip 192.168.0.192

# Terminal 2 — E-stop dashboard

./dashboard.sh --fullscreen

# Unity: File -> Play  (or Build and Run to deploy APK to Quest)

Test input:

# Simulate a joint velocity command without the headset

ros2 topic pub --rate 50 /forward_velocity_controller/commands \

std_msgs/Float64MultiArray '{data: [0.0, 0.0, 0.1, 0.0, 0.0, 0.0]}'

Verify output:

ros2 topic hz /forward_velocity_controller/commands  # expect 50 Hz when Unity Play + controller active

ros2 topic echo /session/status

EXPECTED Moving the right stick forward in RMRC Translate mode causes the UR3e end-effector to move in the same direction. /forward_velocity_controller/commands reports 50 Hz. Dashboard joint angles update live.

5.3.8  Parameter Reference

The following table documents configurable parameters, their valid ranges, and the impact of incorrect values.

| Parameter | Location | Default | Valid Range | Effect of Increasing | What Breaks if Wrong |
| --- | --- | --- | --- | --- | --- |
| linearSpeed | RobotController.cs Inspector | 0.25 m/s | 0.05-0.5 m/s | Faster Cartesian motion per stick deflection | Too high -> overshoot near workspace edges (currently reported too fast — needs field tuning on real hardware) |
| jointJogSpeed | RobotController.cs Inspector | 0.5 rad/s | 0.1-1.5 rad/s | Faster Direct Joint jogging | Too high -> abrupt motion; too low -> unresponsive (currently too slow for wrist joints) |
| handGuideLateralGain | RobotController.cs Inspector | 2.0 | 0.5-5.0 | Controller position delta -> larger EE offset | Too high -> overshoots; too low -> sluggish hand tracking |
| handGuideLateralCapM | RobotController.cs Inspector | 0.15 m/s | 0.05-0.3 m/s | Faster Hand Guide maximum speed | Too high -> operator cannot control; safety risk |
| outputSmoothTime | RobotController.cs Inspector | 0.06 s | 0.02-0.2 s | Smoother but slower response to input | Too high -> sluggish feel; too low -> jittery (exposes sensor noise) |
| poseTimeout | CubePoseSubscriber Inspector | 3.0 s | 0.5-10.0 s | Cubes stay visible longer after last detection | Too low -> cubes flicker; too high -> ghost cubes linger after removal |
| objectScale | CubePoseSubscriber Inspector | 0.04 | 0.01-0.1 | Larger virtual cubes in Unity | Too large -> obscures workspace; too small -> hard to see |
| smoothing | CubePoseSubscriber Inspector | 0.5 | 0.0-0.95 | Smoother cube position but more lag | Too high -> cubes lag behind physical position; 0 = snap to detected pose |
| gripperDeadZone | RobotController.cs Inspector | 0.05 | 0.0-0.2 | Requires harder trigger press to close gripper | Too high -> can't fully close; too low -> resting finger causes grip |
| tableWorldY (MeshCollisionGuard) | MeshCollisionGuard Inspector | N/A (disabled) | Actual table height | Raises the virtual table plane | Wrong value -> collision guard triggers too early or too late; currently disabled |

NOTE Parameters marked with Inspector locations can be modified at runtime in the Unity Editor without recompilation.

5.3.9  Live Configuration Changes

Scenario — "Switch to Direct Joint mode to escape a singularity":

- Press left Menu button to cycle to Direct Joint mode.

- Use right thumbstick Y to jog the selected joint; press A/B to change selected joint.

- Reasoning: near singularities RMRC Jacobian inversion becomes ill-conditioned; Direct Joint lets you manually move individual joints to escape the configuration.

Scenario — "Lower speed for a public demo":

- Inspector: select RobotController GameObject; set linearSpeed from 0.25 to 0.10 m/s.

- Reasoning: the gain multiplies directly into the Cartesian velocity command; halving it gives half-speed for the same stick input.

- Re-enter Play mode or redeploy APK.

Scenario — "Emergency stop without the dashboard":

- Use the physical E-stop on the UR3e teach pendant — halts all motion immediately.

- To resume from the dashboard: hold yellow RESUME button for 5 seconds -> reactivates velocity controllers.

- Reasoning: the burst-zero publish ensures the stop command arrives even with one dropped packet; the 1-second cooldown prevents accidental re-trigger.

5.3.10  Troubleshooting

The ‘Why the Fix Works’ column provides the underlying technical rationale for each resolution.

| Symptom | Likely Cause & Why | How to Diagnose | Fix | Why the Fix Works |
| --- | --- | --- | --- | --- |
| Unity connects but joint angles stay 0.0 | ROS-TCP-Connector v0.7.0 race condition — subscriber registrations lost after Unity Play/Stop without restarting the endpoint | ros2 topic hz /joint_states in terminal — if 125 Hz, ROS is fine; issue is Unity-side | Restart ros_tcp_endpoint (Ctrl+C launch.sh; re-run ./launch.sh); then re-enter Play in Unity | ros_tcp_endpoint v0.7.0 does not re-bind subscriber callbacks across Unity Play/Stop without a full restart |
| Robot drifts when controller is stationary | Sensor noise above effective deadzone threshold | ros2 topic echo /forward_velocity_controller/commands while holding controller still — non-zero = drift | Add or raise software deadzone in RobotController.cs stick input processing before the smoothing filter | A deadzone clips small-magnitude inputs to zero before the low-pass filter, eliminating noise-driven motion |
| RMRC produces sudden joint snap near workspace edge | Near-singular Jacobian — DLS damping may need tuning for this configuration | Look for snapping at full reach or wrist-aligned configs; log Jacobian condition number in UR3eKinematics.cs | Increase the DLS lambda^2 damping factor in UR3eKinematics.cs | Higher lambda bounds the inverse more aggressively near singularities, sacrificing precision but preventing velocity spikes |
| Quest cannot auto-connect to ros_tcp_endpoint | Quest and laptop on different subnets OR beacon.py not running | ping laptop IP from Quest terminal app; check ./launch.sh started beacon | Confirm both on robot router (192.168.0.x); ensure scan range covers the laptop's Wi-Fi IP (192.168.0.101-110) | ROSAutoConnect.cs scans only 192.168.0.101-110; if laptop Wi-Fi IP is outside this range the scan misses it |
| No cubes visible in Unity (perception running) | cube_pose_relay.py not running, or CubePoseSubscriber not subscribed | ros2 topic echo /holoassist/unity/cube_1_pose --once; check Unity console for CubePoseSubscriber subscribe log | Ensure --perception flag passed to launch.sh; check CubePoseSubscriber is attached to a GameObject with robotBase set | cube_pose_relay bridges perception topics to Unity-friendly topics; CubePoseSubscriber needs robotBase to transform ROS→Unity coordinates |
| TF tree disconnected (base_link → camera_link missing) | Camera calibration TF not published — calibration file missing or not loaded | ros2 run tf2_ros tf2_echo base_link camera_link; check ~/.ros2/easy_handeye2/calibrations/ | Run ./calibrate.sh --robot-ip 192.168.0.192 and collect 15-25 samples | The calibration file provides the static transform from base_link to camera_link; without it, tag poses cannot be resolved in the robot frame |
| Port 50002 already in use | Previous UR driver instance still running | fuser 50002/tcp — shows PID of occupying process | fuser -k 50002/tcp then relaunch | The UR driver listens on 50002 for External Control; only one instance can bind the port |

5.3.11  Limitations & Assumptions

Assumptions:

- Operator wears Quest 3 and holds both controllers.

- Robot router provides 192.168.0.x subnet with <50 ms RTT between Quest and laptop.

- UR3e in remote control mode with External Control program running.

- MeshCollisionGuard is disabled — operator must self-monitor proximity to table and self-collision configs.

Known limitations:

- Noticeable latency (~30-80 ms) even on fake hardware — partly unavoidable (WiFi + ROS + Unity 72 Hz).

- linearSpeed (0.25 m/s) too fast and jointJogSpeed (0.5 rad/s) too slow — requires field tuning on real hardware.

- MeshCollisionGuard implemented but disabled — breaks arm control until table height and self-collision margins are calibrated.

- ROS-TCP-Connector v0.7.0 race condition requires restarting ros_tcp_endpoint after Unity Play/Stop.

Future work:

- Haptic feedback on controller when nearing joint limits.

- Tune and re-enable MeshCollisionGuard for real hardware safety.

- Replace ros_tcp_endpoint with zenoh for lower-latency telemetry.

5.3.12  Frequently Asked Questions

Common technical questions and detailed answers regarding the Teleoperation & Interaction subsystem.

Q1:  What is RMRC and how does the Jacobian work?

A1:  RMRC maps desired Cartesian velocity to joint velocities using the geometric Jacobian: dq/dt = J^+ * dx/dt. UR3eKinematics.cs computes the 6x6 Jacobian from current joint angles via FK derivatives, then inverts using damped least-squares (J^T(JJ^T + lambda^2 * I)^-1) so the output stays bounded even at singular configurations.

Q2:  Why three modes instead of one?

A2:  Each is optimal for a different task. RMRC for intuitive Cartesian approach. Direct Joint to escape singular or limit-adjacent configurations where RMRC becomes ill-conditioned. Hand Guide for demonstrating or recording trajectories by physically mirroring controller motion.

Q3:  How does the E-stop guarantee the robot stops?

A3:  The dashboard burst-publishes [0,0,0,0,0,0] ten times, then switches to continuous zero publishing at 50 Hz, then deactivates the controller via ros2 control switch_controllers. The burst covers dropped packets; the controller switch prevents any other node from commanding motion until RESUME is confirmed for 5 s.

Q4:  What is the end-to-end control latency?

A4:  Typical: Unity computes at 50 Hz (20 ms) -> ros_tcp_endpoint serialisation (~2 ms) -> WiFi (~10-30 ms) -> ROS callback -> UR driver executes. Total ~30-80 ms. At 0.25 m/s the robot moves 8-20 mm before the operator sees the result.

Q5:  How does the perception pipeline integrate with teleoperation?

A5:  launch.sh --perception starts the full pipeline: RealSense camera, AprilTag detector, cube_pose_node (computes cube centres from 6 face tags per cube), and cube_pose_relay (bridges to Unity topics). CubePoseSubscriber.cs in Unity subscribes to /holoassist/unity/cube_{1-4}_pose and spawns coloured cubes at detected positions relative to the robot base. The camera-to-robot transform comes from easy_handeye2 calibration (base_link -> camera_link). No workspace board tags needed — cube_pose_node uses base_link as its reference frame directly.

5.4  XR Scene & Visualisation

Group Member: Sebastian Baudille | Software stack: C# (Unity 6.3 LTS) / Meta XR All-in-One SDK v85.0.0 / C++

Package(s): Unity/My project/

6.  System-Level Troubleshooting

6.1 Common System-Level Issues

6.2 Frequently Asked Questions

Q:

A: